//characters for column common cathode and row anode

//If Data is sent in ASCII format then,

//actual character can be obtained by subtracting 32 from the received byte

//Starting byte of 'number_font1[100][5]' array is decimal 32 == SPACE.

char characters_font1[98][5]={   //Rows data by enabling single column

// C1   C2   C3   C4   C5   R1==D0 -- R7==D6

{0X00,0X00,0X00,0X00,0X00}, //Not used

{0X5F,0X00,0X00,0X00,0X00}, //!

{0X07,0X07,0X00,0X00,0X00}, //"

{0X14,0X7F,0X14,0X7F,0X14}, //#

{0X24,0X2A,0X7F,0X2A,0X12}, //$

{0X23,0X13,0X08,0X64,0X62}, //%

{0X36,0X49,0X56,0X20,0X50}, //&

{0X03,0X00,0X00,0X00,0X00}, //'

{0X3E,0X41,0X00,0X00,0X00}, //(

{0X41,0X3E,0X00,0X00,0X00}, //)

{0X2A,0X1C,0X3E,0X1C,0X2A}, //*

{0X08,0X08,0X3E,0X08,0X08}, //+

{0X40,0X30,0X00,0X00,0X00}, //,

{0X08,0X08,0X08,0X08,0X08}, //-

{0X40,0X00,0X00,0X00,0X00}, //.

{0X20,0X10,0X08,0X04,0X02}, ///

{0X3E,0X51,0X49,0X45,0X3E}, //0

{0X44,0X42,0X7F,0X40,0X40}, //1

{0X42,0X61,0X51,0X49,0X46}, //2

{0X41,0X41,0X49,0X4D,0X32}, //3

{0X18,0X14,0X12,0X7F,0X10}, //4

{0X4F,0X49,0X49,0X49,0X31}, //5

{0X3E,0X49,0X49,0X49,0X32}, //6

{0X01,0X01,0X79,0X05,0X03}, //7

{0X36,0X49,0X49,0X49,0X36}, //8

{0X26,0X49,0X49,0X49,0X3E}, //9

{0X14,0X00,0X00,0X00,0X00}, //:

{0X40,0X34,0X00,0X00,0X00}, //;

{0X08,0X14,0X22,0X41,0X00}, //<

{0X14,0X14,0X14,0X14,0X14}, //=

{0X41,0X22,0X14,0X08,0X00}, //>

{0X02,0X01,0X59,0X05,0X02}, //?

{0X3C,0X42,0X59,0X65,0X3E}, //@

{0X7E,0X09,0X09,0X09,0X7E}, //A

{0X7F,0X49,0X49,0X49,0X36}, //B

{0X1C,0X22,0X41,0X41,0X41}, //C

{0X7F,0X41,0X41,0X22,0X1C}, //D

{0X7F,0X49,0X49,0X49,0X41}, //E

{0X7F,0X09,0X09,0X01,0X01}, //F

{0X3E,0X41,0X59,0X49,0X3A}, //G

{0X7F,0X08,0X08,0X08,0X7F}, //H

{0X41,0X41,0X7F,0X41,0X41}, //I

{0X20,0X41,0X3F,0X01,0X00}, //J

{0X7F,0X08,0X14,0X22,0X41}, //K

{0X7F,0X40,0X40,0X40,0X40}, //L

{0X7F,0X02,0X04,0X02,0X7F}, //M

{0X7F,0X04,0X08,0X10,0X7F}, //N

{0X3E,0X41,0X41,0X41,0X3E}, //O

{0X7F,0X09,0X09,0X09,0X06}, //P

{0X3E,0X41,0X71,0X3E,0X40}, //Q

{0X7F,0X09,0X19,0X29,0X46}, //R

{0X26,0X49,0X49,0X49,0X32}, //S

{0X01,0X01,0X7F,0X01,0X01}, //T

{0X3F,0X40,0X40,0X40,0X3F}, //U

{0X1F,0X20,0X40,0X20,0X1F}, //V

{0X7F,0X20,0X10,0X20,0X7F}, //W

{0X63,0X14,0X08,0X14,0X63}, //X

{0X03,0X04,0X78,0X04,0X03}, //Y

{0X61,0X51,0X49,0X45,0X43}, //Z

{0X7F,0X41,0X41,0X00,0X00}, //[

{0X02,0X04,0X08,0X10,0X20}, //\

{0X41,0X41,0X7F,0X00,0X00}, //]

{0X04,0X02,0X01,0X02,0X04}, //^

{0X40,0X40,0X40,0X40,0X40}, //_

{0X01,0X01,0X02,0X00,0X00}, //`

{0X30,0X48,0X48,0X78,0X40}, //a

{0X7F,0X48,0X48,0X48,0X30}, //b

{0X30,0X48,0X48,0X48,0X00}, //c

{0X30,0X48,0X48,0X7F,0X00}, //d

{0X30,0X48,0X58,0X58,0X00}, //e

{0X08,0X7E,0X09,0X09,0X02}, //f

{0X18,0XA4,0XA4,0X7C,0X00}, //g

{0X7F,0X08,0X08,0X08,0X70}, //h

{0X74,0X00,0X00,0X00,0X00}, //i

{0X40,0X80,0X7A,0X00,0X00}, //j

{0X7F,0X10,0X28,0X44,0X00}, //k

{0X7F,0X00,0X00,0X00,0X00}, //l

{0X78,0X08,0X70,0X08,0X70}, //m

{0X78,0X08,0X08,0X70,0X00}, //n

{0X30,0X48,0X48,0X30,0X00}, //o

{0XFC,0X24,0X24,0X18,0X00}, //p

{0X18,0X24,0XFC,0X40,0X20}, //q

{0X28,0X50,0X28,0X00,0X00}, //r

{0X48,0X54,0X54,0X24,0X00}, //s

{0X08,0X7E,0X48,0X20,0X00}, //t

{0X38,0X40,0X40,0X38,0X40}, //u

{0X18,0X20,0X40,0X20,0X18}, //v

{0X38,0X40,0X30,0X40,0X38}, //w

{0X48,0X30,0X30,0X48,0X00}, //x

{0X18,0XA0,0XA0,0X78,0X00}, //y

{0X48,0X68,0X58,0X48,0X00}, //z

{0X08,0X36,0X41,0X41,0X00}, //{

{0X7F,0X00,0X00,0X00,0X00}, //|

{0X41,0X41,0X36,0X08,0X00}, //}

{0X08,0X04,0X08,0X10,0X08}, //~

};

char flag=0,temp,rowdata[1330],temp1,length;

char repeat=9,scrolling=1;

char text[200];

char j=16,i,strlent=0,column,row_data_index;

char n_modules=16; //No.of 8X8 Modules or (No.of columns)/8

int scroll,temp2,k;

void font_verification(void);

void line(const char txt[255]);

void text_framing(void);

void framing(void);

void set_framing(void);

void slower() iv IVT_ADDR_INT0 ics ICS_AUTO

{

EIMSK=0X00;        //Disable External Interrupts

TCCR1B=0X0D;       //Start Timer for One second

Delay_ms(50);

repeat++;          //Increment repetition

repeat%=10;        //Make it less than 10;

if(repeat==0)

{

repeat=9;

Delay_ms(1000);//Delay indication end of speed control

}

}

void faster() iv IVT_ADDR_INT1 ics ICS_AUTO

{

EIMSK=0X00;       //Disable External Interrupts

TCCR1B=0X0D;      //Start Timer for One second

Delay_ms(50);

repeat--;         //Decrement repetition

if(repeat<=1)     //Make it greater than 1;

{

repeat=2;

Delay_ms(1000);//Delay indication end of speed control

}

}

void sec() iv IVT_ADDR_TIMER1_COMPA ics ICS_AUTO

{

EIMSK=0X03;    //Enable Interrupts after Onsecond

TCCR1B=0X00;

}

void spi() iv IVT_ADDR_SPI__STC ics ICS_AUTO

{

flag=1;

}

void main()         //Main

{

PORTB=0XFF;     //Declaring PORTB as output port with high initial values

DDRB=0XFF;

PORTD=0X1F;     //Declaring PORTD as Input port with Pulled Up inputs

DDRD=0XE0;

PORTC=0X00;

DDRC=0XFF;

MSTR_BIT=1;     //Enabling master mode of SPI

SPE_BIT=1;      //Enabling SPI

SPIE_BIT=1;     //Enable SPI Interrupt

OCR1AH=0X3D;

OCR1AL=0X09;

OCIE1A_BIT=1;

EICRA=0X0A;

EIMSK=0X03;

SREG_I_BIT=0;

//line("www.circuitstoday.com");

line("WWW.CIRCUITSTODAY.COM");

//line("ABC");

SREG_I_BIT=1;

if(PIND4_BIT==0)

{

scrolling=0;

for(temp=0;temp<n_modules;temp++)

{

SPDR=0;        //Send empty data to columns

do{}while(flag==0);   //Wait for TX Complete interrupt

flag=0;

}

PORTC1_BIT=1;

Delay_us(2);

PORTC1_BIT=0;

Delay_us(2);

}

column=1;

temp=0;

scroll=0;

temp1=0;

text_framing();

SREG_I_BIT=1;

Delay_us(5);

PORTB0_BIT=0;   //CLK For decade counter=0

PORTB1_BIT=0;   //Disable Shift Register Outputs

PORTC0_BIT=0;   //Master Reset of decade counter=0

PORTC1_BIT=0;   //Disable STROBE Input for shift register

//font_verification();

while(1)

{

for(temp2=0;temp2<5*repeat;temp2++)

{

PORTC0_BIT=1;  //Issue master reset signal to decade counter

Delay_us(2);

PORTC0_BIT=0;

for(column=1;column<=8;column++)

{

for(temp=0;temp<n_modules;temp++)

{

row_data_index=column+(8*temp);

row_data_index+=scroll;

if(row_data_index<=length)

{

SPDR=rowdata[row_data_index];

}

else

{

SPDR=0X00;

}

do{}while(flag==0);   //Wait for TX Complete interrupt

flag=0;

}

PORTC1_BIT=1;      //STROBE Input ==> Negative edge

Delay_us(2);

PORTC1_BIT=0;

Delay_us(2);

PORTB1_BIT=1;      //Enable Shift Register Outputs

Delay_us(250);     //Wait=Refreshing ON time

PORTB1_BIT=0;      //Disable Shift Register Outputs

Delay_us(2);

PORTB0_BIT=1;      //Issue clock pulse to decade counter

Delay_us(2);

PORTB0_BIT=0;

Delay_us(2);

}

}

if(scrolling==1)

{

scroll++;

scroll%=length;

}

}

}

void text_framing(void)

{

char j,k,empty_column;

int c;

if(scrolling==0)

{

c=1;

}

else

{

for(c=1;c<=n_modules*8;c++)//Issue of empty data to start the text from Right end

{

rowdata[c]=0X00;

}

//c=1;

}

for(j=0,k=0;text[j]!=13;c++)

{

rowdata[c]=characters_font1[text[j]-32][k];

k++;

if(rowdata[c]==0X00)       //End of character's data

{

empty_column=1;

if(text[j]!=32)        //End of character's data

{

k=5;

}

}

if(k==5)

{

j++;   //Go to next chaaracter

k=0;   //Reset character's array index

//Inserting space between two characters in a word

if(empty_column==0)    //Character is 5-Columned

{

c++;

rowdata[c]=0X00;   //Insert empty column

}

else   //Character itself contains empty column at the end

{

empty_column=0;

}

}

}

length=c;   //Text contains c no.of columns including initial empty data

for(j=0;j<8*n_modules;j++,c++)

{

rowdata[c]=0X00;

}

}

void font_verification(void)

{

for(j=0;j<97;j++)

{

for(k=0;k<=1000;k++)

{

PORTC0_BIT=1;  //Issue master reset signal to decade counter

Delay_us(5);

PORTC0_BIT=0;

for(i=0;i<=4;i++)

{

SPDR=characters_font1[j][i];

do{}while(flag==0);   //Wait TX Complete interrupt

flag=0;

PORTC1_BIT=1;      //STROBE Input ==> Negative edge

Delay_us(2);

PORTC1_BIT=0;

Delay_us(2);

PORTB1_BIT=1;     //Enble Shift register outputs

Delay_us(200);   //Refreshing ON-Time

PORTB1_BIT=0;     //Disable Shift register outputs

PORTB0_BIT=0;  //Issue clock signal to decade counter

Delay_us(5);

PORTB0_BIT=1;

}

}

}

}

void line(const char txt[255])

{

char indx;

for(indx=0;txt[indx]!='\0';indx++)

{

text[indx]=txt[indx];

}

text[indx]=13;

}