//C-Program for DOT MATRIX DISPLAY Clock

//16 MHz Clock frequency, ATMEGA 8 Microcontroller

//                           C1 C2 C3 C4 C5    C1 C2 C3 C4 C5

//BIT0 = ROW1           *    *                 0    1    1   0   0

//BIT1 = ROW2      *               *           1     0    0   1  0

//BIT2 = ROW3      *               *           1     0    0   1  0

//BIT3 = ROW4      *               *           1     0    0   1  0

//BIT4 = ROW5      *               *           1     0    0   1  0

//BIT5 = ROW6      *               *            1    0    0   1  0

//BIT6 = ROW7            *   *                  0    1    1   0  0

//characters for column common cathode and row anode

char number_font1[10][5]={   //Rows data by enabling single column

// C1   C2   C3   C4   C5   R1==D0 -- R7==D6

{0X3E,0X41,0X41,0X3E,0X00}, //0

{0X44,0X42,0X7F,0X40,0X00}, //1

{0X62,0X51,0X49,0X46,0X00}, //2

{0X41,0X49,0X4D,0X32,0X00}, //3

{0X18,0X14,0X12,0X7F,0X00}, //4

{0X4F,0X49,0X49,0X31,0X00}, //5

{0X3E,0X49,0X49,0X32,0X00}, //6

{0X01,0X01,0X79,0X07,0X00}, //7

{0X36,0X49,0X49,0X36,0X00}, //8

{0X26,0X49,0X49,0X3E,0X00}, //9

};

char i=0,j,k,flag=0,temp,time[6]={1,2,0,0,0,0},rowdata[32],shift,frame=0;

char separator=0,position,set=0,rtc_reg[8];

void framing(void);

void set_framing(void);

void twi_strt(void);

void twi_stp(void);

void set_time(void);

void read_time(void);

void change() iv IVT_ADDR_INT0 ics ICS_AUTO

{

if((PIND&0X07)==0X03)       //If set/change button alone is pressed

{

set++;                  //Initially set=0

if(set==1)  //Button is pressed for the first time so change time

{

GICR=0X40;

time[0]=1;

time[1]=0;

time[2]=1;

time[3]=0;

time[4]=1;

time[5]=0;

set_framing();

}

else      //Button is pressed for the second time so time is set

{

rtc_reg[0]=time[4]*0X10+time[5];

rtc_reg[1]=time[2]*0X10+time[3];

rtc_reg[2]=(time[0]&0X07)*0X10+time[1];

framing();

set_time();

position=0;

set=0;

GICR=0XC0;

}

}

else if((PIND&0X07)==0X01)    //If Shift button is pressed

{

if(set==1)      //Avoiding un wanted operation if shift key is pressed

{

position++;    //Increment position variable

set_framing(); //Go to set framing function

}

}

else if((PIND&0X07)==0X02)

{

if(set==1)    //Avoiding un wanted operation if adjust key is pressed

{

switch(position)

{

case 0:{//thrs in 12 hrs format

time[0]++;     //Ten hours will be either 0 or 1

time[0]%=2;

}break;

case 1:{//hrs in 12 hrs format

time[1]++;     //Depending on ten hours value

if(time[0]==0)  //Limit hours value

{

time[1]%=10;

}

else

{

time[1]%=3;

}

}break;

case 2:{//tmint

time[2]++;     //Ten minutes value will be 0 to 5

time[2]%=6;

}break;

case 3:{//mint

time[3]++;

time[3]%=10;

}break;

case 4:{//tsec            //Ten seconds value will be 0 to 5

time[4]++;

time[4]%=6;

}break;

case 5:{//sec

time[5]++;

time[5]%=10;

}break;

}

set_framing();        //Go to set framing with the updated values

}

}

}

void rtc() iv IVT_ADDR_INT1 ics ICS_AUTO

{

SREG_I_BIT=0;

GICR=0X00;

separator++;

separator%=2;

read_time();

framing();

GICR=0XC0;

SREG_I_BIT=1;

}

void spi() iv IVT_ADDR_SPI__STC ics ICS_AUTO

{

flag=1;

}

void main()

{

PORTB=0XFF;     //Declaring PORTB as output port with high initial values

DDRB=0XFF;

PORTD=0XFF;     //Declaring PORTD as Input port with Pulled Up inputs

DDRD=0X00;

DDRC=0XFF;

MSTR_BIT=1;     //Enabling master mode of SPI

SPE_BIT=1;      //Enabling SPI

SPIE_BIT=1;     //Enable SPI Interrupt

TWBR=0X48;      //TWI Baud rate

read_time();

framing();

rtc_reg[7]=0X10;

set_time();

PORTC0_BIT=1;  //Issue clock signal to decade counter

Delay_us(5);

PORTC0_BIT=0;

MCUCR=0X0A;

GICR=0XC0;

SREG_I_BIT=1;

Delay_us(5);

shift=4;

while(1)

{

//Negation of row data is because, we are using Column anode in proteus

SPDR=~rowdata[8*(shift-1)+frame];

do{}while(flag==0);   //Wait TX Complete interrupt

flag=0;

shift--;              //According to logic Hint:8*4=32 columns

if(shift==0)          //So shift is run from 4 to 1

{                     //4-Bytes of data should be sent for each frame

shift=4;

frame++;          //Increment the frame

PORTB1_BIT=1;  //Enable output

Delay_ms(2);   //Wait this can be varied

PORTB1_BIT=0;  //Disable outputs of registers

PORTB0_BIT=0;  //Issue clock signal to decade counter

Delay_us(5);

PORTB0_BIT=1;

if(frame==8)

{

frame=0;   //frame runs from 0 to 7

PORTC0_BIT=1;  //Issue master reset signal to decade counter

Delay_us(5);

PORTC0_BIT=0;

}

}

}

}

void framing(void)

{

char i,j,k;

for(i=0,j=0,k=0;i<=31;i++)

{

if((i!=10)&&(i!=21))

{

rowdata[i]=number_font1[time[j]][k];

k++;

if(k==5)

{

j++;

k=0;

}

}

else

{

if(separator==0)

{

rowdata[i]=0X14;

}

else

{

rowdata[i]=0X00;

}

}

}

}

void set_framing(void)

{

char i,j,k,l;

switch(position)

{

case 0:{

l=4;

}break;

case 1:{

l=9;

}break;

case 2:{

l=15;

}break;

case 3:{

l=20;

}break;

case 4:{

l=26;

}break;

case 5:{

l=31;

}break;

}

for(i=0,j=0,k=0;i<=l;i++)

{

if((i!=10)&&(i!=21))

{

rowdata[i]=number_font1[time[j]][k];

k++;

if(k==5)

{

j++;

k=0;

}

}

else

{

rowdata[i]=0X00;

}

}

for(i=l+1,j=0,k=0;i<=31;i++)

{

if((i!=10)&&(i!=21))

{

rowdata[i]=0X00;

k++;

if(k==5)

{

j++;

k=0;

}

}

else

{

rowdata[i]=0X00;

}

}

}

void twi_strt(void)

{

TWCR=0XA4;                                             //send start command

do{}while(TWINT_BIT==0);              //wait untill TX complete falg is set

}

void twi_stp(void)

{

TWCR=0X94;

do{}while(TWINT_BIT==1);

Delay_us(15);

}

void set_time(void)

{

rtc_reg[temp]=0X10;

twi_strt();

TWDR=0XD0;                          //send RTC Address

TWCR=0X84;

do{}while(TWINT_BIT==0);           //wait untill TX complete falg is set

TWDR=0X00;                          //send register address

TWCR=0X84;

do{}while(TWINT_BIT==0);       //wait untill TX complete falg is set

for(temp=0;temp<=7;temp++)

{

TWDR=rtc_reg[temp];

TWCR=0X84;

do{}while(TWINT_BIT==0);       //wait untill TX complete falg is set

}

}

void read_time(void)

{

char temp_read;

for(temp_read=0;temp_read<=2;temp_read++)

{

twi_strt();         // issue TWI start signal

TWDR=0XD0;              // send byte via TWI (device address + Write)

TWCR=0X84;

do{}while(TWINT_BIT==0);

TWDR=temp_read;        //Send address of the byte to be read

TWCR=0X84;

do{}while(TWINT_BIT==0);

twi_strt();

TWDR=0XD1;                // send byte (device address + Read)

TWCR=0X84;

do{}while(TWINT_BIT==0);

TWEA_BIT=0;         // read data (NO acknowledge)

do{}while(TWINT_BIT==0);   //wait until data byte is received

rtc_reg[temp_read]=TWDR;

twi_stp();

}

//Issuing register values to time variables

for(temp_read=0;temp_read<=5;temp_read++)

{

if((temp_read%2)==0)

{

time[5-temp_read]=rtc_reg[temp_read/2]%16;

}

else

{

time[5-temp_read]=rtc_reg[temp_read/2]/16;

}

}

}